PARTITION COEFFICIENTS FOR

METALS

IN SURFACE WATER, SOIL, AND WASTE

by

Jerry D. Allison, Ph.D.1, 2

Terry L. Allison, P.G.2

1HydroGeoLogic, Inc

1155 Herndon Parkway, Suite 900

Herndon, VA 20170

2Allison Geoscience Consultants, Inc.

3920 Perry Lane

Flowery Branch, GA 30542

Work Assignment Manager:	 	Robert B. Ambrose, Jr., P.E.

Contract No. 68-C6-0020		U.S. Environmental Protection Agency

Office of Research and Development

Athens, GA 30605

U.S. Environmental Protection Agency

Office of Research and Development

National Exposure Research Laboratory

Ecosystems Research Division

Athens, GA 30605

July 2005

ABSTRACT

This report presents metal partition coefficients for the surface water pathway and for the source model used in the Multimedia, Multi-pathway, Multi-receptor Exposure and Risk Assessment (3MRA) technology under development by the U.S. Environmental Protection Agency.  Partition coefficients values are presented for partitioning between soil and water; partitioning between the suspended sediment load and the water in streams, rivers, and lakes; partitioning between riverine or lacustrine sediment and its porewater; and partitioning between dissolved organic carbon (DOC) and the inorganic solution species in the water of streams, rivers, and lakes.  Some partition coefficients are also presented to represent metal partitioning between the solid phase of waste and its associated leachate.  Partition coefficients are presented for antimony (Sb), arsenic (As), barium (Ba), beryllium (Be), cadmium (Cd), chromium (Cr), cobalt (Co), copper (Cu), lead (Pb), molybdenum (Mo), mercury (Hg), methylated mercury (CH3Hg), nickel (Ni), selenium (Se), silver (Ag), thallium (Tl), tin (Sn), vanadium (V), and zinc (Zn).

A two-phase approach was used in developing the needed partition coefficients.  In the first-phase, a literature survey was performed to determine the range and statistical distribution of values that have been observed in field studies.  This included the collection of published partition coefficients for any of the metals in any of the environmental media of interest, or our estimation of partition coefficients from reported metal concentration data when feasible.  In the second-phase effort, statistical methods, geochemical speciation modeling, and expert judgement were used to provide reasonable estimates of those partition coefficients not obtained from our literature search and data processing.  The report concludes with a discussion of the many sources of uncertainty in the reported metal partition coefficients.

| Table 3 Metal partition coefficients (log Kd) in L/kg for soil/soil water. Values in italics were estimated by regression or from MINTEQA2 results. An entry of “log-normal” indicates that the sample data gave a positive result in the Shapiro-Wilk test for normality of the log-transformed data. An entry of “log-normal assumed” in parentheses means that data were not sufficient to establish the distribution, but log-normal has been assumed. Relative confidence in the data is indicated by the CL value of 1 to 4 (1=highest, 4=lowest). An entry of “---” for the median occurs where regression equations were used to estimate the mean, minimum, and maximum values and no estimate was made for the median. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Metal | Median | Mean | Std. Dev. | Min | Max | Comments |
| Ag (I) | 2.6 | 2.6 | 0.8 | 1.0 | 4.5 | From literature data (raw, n=21); log-normal; CL=1 |
| Asa | 3.4 | 3.2 | 0.7 | 0.3 | 4.3 | From literature data (raw, n=21); (log-normal assumed); oxidation state usually not specified in literature; CL=2 |
| Ba(II) | --- | 2.0 | 0.7 | 0.7 | 3.4 | Suspended matter Kd regression equation for mean; (log-normal assumed); CL=2 |
| Be(II) | --- | 2.2 | 1.0 | 1.7 | 4.1 | Suspended matter Kd regression equation for mean; (log-normal assumed); CL=3 |
| Cd(II) | 2.9 | 2.7 | 0.8 | 0.1 | 5.0 | From literature data (edited, n=37); log-normal; CL=1 |
| Co(II) | 2.1 | 2.1 | 1.2 | -1.2 | 4.1 | From literature data (raw, n=11); log-normal; CL=1 |
| Cr(III) | 3.9 | 3.8 | 0.4 | 1.0 | 4.7 | From literature data (raw, n=22); log-normal; CL=2 |
| Cr(VI) | 1.1 | 0.8 | 0.8 | -0.7 | 3.3 | From literature data (raw, n=24); (log-normal assumed); CL=2 |
| Cu(II) | 2.7 | 2.5 | 0.6 | 0.1 | 3.6 | From literature data (raw, n=20); log-normal; CL=1 |
| Hg(II) | 3.8 | 3.6 | 0.7 | 2.2 | 5.8 | From literature data (raw, n=17); log-normal; CL=1 |
| MeHg | 2.8 | 2.7 | 0.6 | 1.3 | 4.8 | From literature data (raw, n=11); log-normal; CL=2 |
| Mo(VI) | 1.1 | 1.3 | 0.6 | -0.4 | 2.7 | From literature data (raw, n=5); (log-normal assumed); oxidation state not always specified in literature data; CL=3 |
| Ni(II) | 3.1 | 2.9 | 0.5 | 1.0 | 3.8 | From literature data (raw, n=19); log-normal; CL=1 |
| Pb(II) | 4.1 | 3.7 | 1.2 | 0.7 | 5.0 | From literature data (edited, n=31); (log-normal assumed); CL=2 |
| Sbb | --- | 2.3 | 1.1 | 0.1 | 2.7 | From literature data (mean is the average of several reported mean values, n=5); (log-normal assumed); CL=4 |
| Se(IV)c | 1.4 | 1.3 | 0.4 | -0.3 | 2.4 | From literature data (edited, n=11); (log-normal assumed); CL=2 |
| Se(VI) | --- | -0.2 | 1.1 | -2.0 | 2.0 | Mean estimated from MINTEQA2 result; (log-normal assumed); min, max from expert judgement; CL=4 |
| Sn(II) | --- | 2.7 | 0.7 | 2.1 | 4.0 | From literature data; (log-normal assumed); CL=3 |
| Tl(I) | --- | 0.5 | 0.9 | -1.2 | 1.5 | Estimated from MINTEQA2 result; (log-normal assumed); CL=4 |
| V(V) | --- | 1.7 | 1.5 | 0.5 | 2.5 | Mean, min, max from suspended matter Kd regression equation; (log-normal assumed); CL=4 |
| Zn(II) | 3.1 | 2.7 | 1.0 | -1.0 | 5.0 | From literature data (raw, n=21); (log-normal assumed); CL=1 |
| CN- | --- | 0.7 | 1.6 | -2.4 | 1.3 | Estimated from MINTEQA2 result; (log-normal assumed); CL=4 |

a	Published partitioning data for As does not allow differentiation of As(III) and As(V).  It is probable that published values represent results involving both oxidation states.

b	Published partitioning data for Sb is rare and does not allow differentiation of Sb(III) and Sb(V).

c	Positive result in Shapiro-Wilk test for normality of data not log-transformed.  But sample size is small and data may not be very representative.

| Table 4 Metal partition coefficients (log Kd) in L/kg for sediment/porewater. Values in italics were estimated by regression or from MINTEQA2 results. An entry of “log-normal” indicates that the sample data gave a positive result in the Shapiro-Wilk test for normality of the log-transformed data. An entry of “log-normal assumed” in parentheses means that data were not sufficient to establish the distribution, but log-normal has been assumed. Relative confidence in the data is indicated by the CL value of 1 to 4 (1=highest, 4= lowest). An entry of “---” for the median occurs where regression equations were used to estimate the mean, minimum, and maximum values and no estimate was made for the median. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Metal | Median | Mean | Std. Dev. | Min | Max | Comments |
| Ag(I) | --- | 3.6 | 1.1 | 2.1 | 5.8 | Mean from soil Kd regression equation; (log-normal assumed); min, max from literature data; CL=3 |
| Asa | 2.2 | 2.4 | 0.7 | 1.6 | 4.3 | From literature data; log-normal; oxidation state not specified in literature data; CL=2 |
| Ba(II) | --- | 2.5 | 0.8 | 0.9 | 3.2 | Mean, min, max from suspended matter Kd regression equation; (log-normal assumed); CL=3 |
| Be(II) | --- | 2.8 | 1.9 | 0.8 | 6.5 | Mean, min, max from suspended matter Kd regression equation; (log-normal assumed); CL=3 |
| Cd(II) | 3.7 | 3.3 | 1.8 | 0.5 | 7.3 | From literature data (n=14, edited); log-normal; CL=1 |
| Co(II) | --- | 3.1 | 1.0 | 2.9 | 3.6 | Mean from soil Kd regression equation; (log-normal assumed); min, max from literature data; CL=3 |
| Cr(III) | --- | 4.9 | 1.5 | 1.9 | 5.9 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |
| Cr(VI) | --- | 1.7 | 1.4 | 0.0 | 4.4 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |
| Cu(II) | 4.1 | 3.5 | 1.7 | 0.7 | 6.2 | From literature data (raw, n = 12); log-normal; CL=1 |
| Hg(II) | --- | 4.9 | 0.6 | 3.8 | 6.0 | From literature data (raw, n=2); (log-normal assumed); CL=2 |
| MeHg | --- | 3.9 | 0.5 | 2.8 | 5.0 | From literature data (edited, n=2); (log-normal assumed); CL=2 |
| Mo(VI) | --- | 2.5 | 0.8 | 0.4 | 3.7 | Mean from literature data (reported mean value with oxidation state not specified); (log-normal assumed); min, max from soil Kd regression equation; CL=4 |
| Ni(II) | --- | 3.9 | 1.8 | 0.3 | 4.0 | Mean from soil Kd regression equation; (log-normal assumed); min, max from literature data; CL=3 |
| Pb(II) | 5.1 | 4.6 | 1.9 | 2.0 | 7.0 | From literature data (edited, n=14); log-normal; CL=1 |
| Sbb | --- | 3.6 | 1.8 | 0.6 | 4.8 | From literature data (reported mean value); (log-normal assumed); CL=4 |
| Se(IV) | --- | 3.6 | 1.2 | 1.0 | 4.0 | Mean from literature data (reported mean value); (log-normal assumed); min, max from expert judgement; CL=4 |
| Se(VI) | --- | 0.6 | 1.2 | -1.4 | 3.0 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |
| Sn(II) | --- | 3.7 | 0.7 | 3.1 | 5.1 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=3 |
| Tl(I) | --- | 1.3 | 1.1 | -0.5 | 3.5 | Mean, min from soil Kd regression equation; (log-normal assumed); max from literature data; CL=4 |
| V(V) | --- | 2.1 | 0.9 | 0.4 | 3.2 | Mean, min, max from suspended matter Kd regression equation; (log-normal assumed); CL=4 |
| Zn(II) | 4.8 | 4.1 | 1.6 | 1.5 | 6.2 | From literature data (edited, n=13); (log-normal assumed); CL=1 |
| CN- | --- | 1.6 | 1.7 | -1.8 | 2.2 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |

a	Published metal partitioning data does not allow differentiation of As(III) and As(V).  It is probable that the data presented include results for both oxidation states.

b	Published partitioning data for Sb is rare and does not allow differentiation of Sb(III) and Sb(V).

| Table 5 Metal partition coefficients (log Kd) in L/kg for suspended matter/water. Values in italics were estimated by regression or from MINTEQA2 results. An entry of “log-normal” indicates that the sample data gave a positive result in the Shapiro-Wilk test for normality of the log-transformed data. An entry of “log-normal” in parentheses means that data were not sufficient to establish the distribution, but log-normal has been assumed. Relative confidence in the data is indicated by the CL value of 1 to 4 (1=highest, 4= lowest). An entry of “---” for the median occurs where regression equations were used to estimate the mean, minimum, and maximum values and no estimate was made for the median. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Metal | Median | Mean | Std. Dev. | Min | Max | Comments |
| Ag(I) | 5.2 | 5.2 | 0.6 | 4.4 | 6.3 | From literature data (edited, n=9); log-normal; CL = 2 |
| Asa | 4.0 | 3.9 | 0.5 | 2.0 | 6.0 | From literature data (raw, n=25); (log-normal assumed); oxidation state not specified in the literature data; CL=2 |
| Ba(II) | 4.0 | 4.0 | 0.4 | 2.9 | 4.5 | From literature data (raw, n=14); log-normal; CL=2 |
| Be(II) | 4.1 | 4.2 | 0.7 | 2.8 | 6.8 | From literature data (raw, n=17); log-normal; CL=2 |
| Cd(II) | 5.0 | 4.9 | 0.6 | 2.8 | 6.3 | From literature data (edited, n=38); log-normal; CL=1 |
| Co(II) | 4.7 | 4.8 | 0.8 | 3.2 | 6.3 | From literature data (edited, n=20); log-normal; CL=1 |
| Cr(III) | 5.1 | 5.1 | 0.4 | 3.9 | 6.0 | From literature data (raw, n=25); log-normal; assumes unspecified oxidation state is (III); CL=2 |
| Cr(VI) | --- | 4.2 | 0.5 | 3.6 | 5.1 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |
| Cu(II) | 4.7 | 4.7 | 0.4 | 3.1 | 6.1 | From literature data (edited, n=42); log-normal; CL=1 |
| Hg(II)b | 5.3 | 5.3 | 0.4 | 4.2 | 6.9 | From literature data (edited, n=26); log-normal; CL=1 |
| MeHg | --- | 4.9 | 0.7 | 4.2 | 6.2 | Mean from soil Kd regression equation; (log-normal assumed); min, max from literature data; CL=3 |
| Ni(II)b | 4.3 | 4.4 | 0.4 | 3.5 | 5.7 | From literature data (edited, n=25); log-normal; CL=1 |
| Mo(VI) | --- | 4.4 | 1.0 | 3.7 | 4.9 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |
| Pb(II)c | 5.7 | 5.7 | 0.4 | 3.4 | 6.5 | From literature data (edited, n=38); (log-normal assumed); CL=1 |
| Sbd | --- | 4.8 | 0.5 | 3.9 | 4.9 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |
| Se(IV) | --- | 4.4 | 0.4 | 3.8 | 4.8 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |
| Se(VI) | --- | 3.8 | 1.0 | 3.1 | 4.6 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |
| Sn(II) | --- | 4.9 | 0.8 | 4.7 | 6.3 | Mean, min from soil Kd regression equation; (log-normal assumed); max from literature data; CL=4 |
| Tl(I) | --- | 4.1 | 1.0 | 3.0 | 4.5 | Mean from soil Kd regression equation; (log-normal assumed); other parameters from expert judgement; CL=4 |
| V(V) | --- | 3.7 | 0.6 | 2.5 | 4.5 | Mean from literature data (raw, n=5); (log-normal assumed); min, max from expert judgement; oxidation state not always specified in literature; CL=3 |
| Zn(II) | 5.1 | 5.0 | 0.5 | 3.5 | 6.9 | From literature data (edited, n=47); log-normal; CL=1 |
| CN- | --- | 4.2 | 0.6 | 3.0 | 4.4 | Mean, min, max from soil Kd regression equation; (log-normal assumed); CL=4 |

a	Positive result for Shapiro-Wilk test for normality of data not log-transformed.  Published metal partitioning data does not allow differentiation of As(III) and As(V).  It is probable that the data represented include results for both oxidation states.

b	Failed Shapiro-Wilk test for normality of log-transformed data, but passed the Kolmogorov-Smirnov test and histogram exhibits log-normal character.

c	Failed Shapiro-Wilk and the Kolmogorov-Smirnov test for normality of log-transformed data, but histogram exhibits log-normal character

d	Published partitioning data for Sb is rare and does not allow differentiation of Sb(III) and Sb(V).

| Table 6 Metal partition coefficients (log Kd) in L/kg for partitioning between DOC and inorganic solution species. Values in italics were estimated by regression or from MINTEQA2 results. Log-normal distributions are assumed. Relative confidence in the data is indicated by the CL value of 1 to 4 (1=highest, 4=lowest). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Metal | Mean | Std. Dev. | Min | Max | Comment |
| Ag(I) | 2.5 | 1.0 | 1.5 | 4.5 | Mean estimated from MINTEQA2 results; other parameters from expert judgement; (log-normal assumed); CL=3 |
| As | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); (log-normal assumed); CL=4 |
| Ba(II) | 3.6 | 1.0 | 2.5 | 4.0 | Mean estimated from MINTEQA2 results, values for other parameters from expert judgement; (log-normal assumed); CL=3 |
| Be(II) | 2.1 | 1.0 | 1.1 | 3.8 | All parameters estimated from MINTEQA2 results; CL=3 |
| Cd(II) | 3.8 | 0.9 | 2.0 | 5.5 | Mean estimated from MINTEQA2 results; min, max from expert judgement; CL=3 |
| Co(II) | 3.8 | 0.9 | 2.0 | 5.5 | Mean estimated from MINTEQA2 results; min, max from expert judgement; CL=3 |
| Cr(III) | 1.1 | 1.6 | -0.6 | 4.3 | Mean estimated from MINTEQA2 results; min, max from expert judgement; CL=4 |
| Cr(VI) | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); CL=4 |
| Cu(II) | 5.4 | 1.1 | 2.5 | 7.0 | From literature data (raw, n=17); (log-normal assumed); CL=2 |
| Hg(II) | 5.4 | 1.2 | 3.0 | 6.0 | Mean from literature data (raw, n=3); (log-normal assumed); min, max from expert judgement; CL=4 |
| MeHg | 5.0 | 1.1 | 2.8 | 5.5 | Mean, min, max estimated based on relative Kd’s of Hg(II) and MeHg for suspended matter and Hg(II) Kd with DOC; (log-normal assumed); CL=4 |
| Ni(II) | 3.7 | 0.9 | 1.9 | 5.4 | Mean estimated from MINTEQA2 results; min, max from expert judgement; (log-normal assumed); CL=3 |
| Mo(VI) | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); CL=4 |
| Pb(II) | 4.9 | 0.5 | 3.8 | 5.6 | From literature data (raw, n=9); (log-normal assumed); CL=2 |
| Sb | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); CL=4 |
| Se(IV) | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); CL=4 |
| Se(VI) | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); CL=4 |
| Sn(II) | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); CL=4 |
| Tl(I) | 1.6 | 1.0 | 0.0 | 3.0 | Mean estimated from MINTEQA2, values for other parameters from expert judgement; (log-normal assumed); CL=4 |
| V(V) | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); CL=4 |
| Zn(II) | 5.1 | 0.7 | 4.6 | 6.4 | From literature data (raw, n=9); (log-normal assumed); CL=3 |
| CN- | 2.0 | 1.0 | 0.0 | 3.0 | No data, values from expert judgement (conservative); (log-normal assumed); CL=4 |